Phase 2: Midterm Progress & Troubleshooting

0. Student Information

Name: 안형준
Student ID: 20221250

1. Initial Training & Loss Analysis

  - Problem and model recap

∂θ∂τ=∂2θ∂X2-βθ+Siτ2gX,  β=hPL2kA,  S=q0Iref2L2kΔTref

then  h,q0 are recovered after training.

• Boundary/initial conditions(both axial ends adiabatic):

θX,0=0,  θX0,τ=0, θX1,τ=0

• Convection enters as the distributed sink -βθ (side-surface cooling), not as a boundary condition.

• gX=exp-(X-0.30)220.102 : Gaussian heat source centered at X=0.30

• iτ=ItIref

  - Baseline PINN configuration

| Setting | Baseline value |
| --- | --- |
| Backend / device | PyTorch backend, CPU forced (PINN_FORCE_CPU=1) — MPS disabled on M1 |
| Network | dde.nn.FNN([2,32,32,32,1], "tanh", "Glorot normal") |
| Depth × width | 3 × 32 |
| Optimizer / lr | Adam, 1e-3 (optional L-BFGS polishing) |
| Iterations | 100,000 |
| Collocation | num_domain=512, num_boundary=64, num_initial=64, num_test=200 |
| Sensors | 3 sensors at X=0.25, 0.55, 0.85; sensor_time_points=41 (123 points) |
| Synthetic data | generated in the FDM reference solver section of the notebook. |
| Current profile | 0–300 s on (I = I_ref), 300–600 s off, 600–900 s on (I = I_ref), 900–1200 s off |

  - Loss components and weighting

| Loss term | Meaning | Weight (baseline) |
| --- | --- | --- |
| Loss_PDE | MSE of PDE residual f | 5 |
| Loss_IC | θX,0=0 mismatch | 10 |
| Loss_BC (left+right) | adiabatic θX=0 at X=0,1 | 10 + 10 |
| Loss_data | sensor data mismatch | 200 |

weight vector = [PDE, IC, lBC, rBC, data] = [5, 10, 10, 10, 200]

  - Loss-weight balancing study

| Case | Weights [PDE,IC,lBC,rBC,data] | beta err | S err | Rel L2 |
| --- | --- | --- | --- | --- |
| A1 | [5,10,10,10,100] | -0.199% | -0.777% | 5.46e-03 |
| A2 | [5,10,20,20,100] | -0.355% | -0.571% | 7.32e-03 |
| A3 | [10,10,10,10,100] | +0.472% | +0.042% | 4.41e-03 |
| A4 | [5,10,10,10,200] | -0.005% | -0.351% | 4.60e-03 |
| A5 | [10,10,10,10,200] | +0.508% | -0.442% | 5.76e-03 |

A1 [5,10,10,10,100]: used a moderate PDE weight and a sensor-data weight of 100 as the starting configuration. Recovery was reasonable, but S error remained relatively high (-0.777%).

A2 [5,10,20,20,100]: increased the two boundary-condition weights. This did not help — test loss and Rel L2 got worse, so over-weighting the adiabatic BCs is counter-productive here.

A3 [10,10,10,10,100]: raised the PDE weight while keeping the IC/BC/data weights unchanged. Best full-field Rel L2 of the set and near-zero S error, but beta error rose to +0.472%.

A4 [5,10,10,10,200]: doubled the sensor-data weight to 200 while keeping PDE moderate. Decisive change — near-perfect beta recovery (-0.005%) and the lowest MAE. Selected as baseline.

A5 [10,10,10,10,200]: additionally raised the PDE weight on top of A4. No meaningful gain (beta error back up to +0.508%), confirming that the data weight — not the PDE weight — drives parameter identifiability.

In this inverse setting a high sensor-data weight is what makes h and q0 identifiable, while the PDE/BC weights mainly affect full-field smoothness. A4 is therefore taken as the baseline; its convergence curves and final summary follow in 1.5 and 1.6.

  - Loss convergence curves and stability analysis

Figure 1.5 — Loss convergence for the selected A4 baseline (weights [5, 10, 10, 10, 200], 3×32 network, 100,000 Adam iterations). Best train/test loss = 1.97×10⁻³ / 8.37×10⁻³ at step 98,000

All plotted loss components decrease roughly by three to five orders of magnitude and settle onto a stable plateau, indicating a converged, non‑divergent training run. The selected A4 baseline reaches train loss = 1.97×10⁻³ and test loss = 8.37×10⁻³ at its best step (98,000 / 100,000), consistent with the parameter and full‑field errors reported for A4 in Section 1.6 (h err −0.005 %, q0 err −0.351 %, Rel L² 4.60×10⁻³).

Early transient (0 – ~5,000 iters): Loss_Total drops sharply from roughly 2.5×10² to below 1. Loss_BC shows a brief initial rise (to ~5×10⁻¹) before falling; this is expected, as the network first fits the dominant PDE/IC/data terms and only then drives the adiabatic boundaries to zero. It is a transient, not an instability.

Main descent (~5,000 – 50,000 iters): All curves decrease smoothly and roughly in parallel on the log scale, the expected Adam behaviour. Only one minor bump appears (around iter 26,000) and recovers immediately.

Plateau (>~50,000 iters): Loss_Total levels off near 2×10⁻³ and Loss_PDE near 1–2×10⁻³, matching the converged train loss (1.97×10⁻³) reported for A4 in Section 1.6. Intermittent upward spikes occur — the largest peaks at about 2.4×10⁻² near iter 82,000, coinciding with rises in Loss_PDE — but each spike decays back to the plateau within a few hundred iterations. These oscillations are attributed to Adam adaptive-update dynamics and the competing effects between PDE residual minimization and sparse sensor-data fitting; there is no upward drift and no sustained high plateau.

Term balance: Loss_PDE tracks Loss_Total almost exactly, so the PDE residual is the binding (hardest) term, while Loss_BC sits about one to two orders of magnitude lower throughout — the adiabatic ends θX = 0 are the easiest condition to satisfy. With the [5, 10, 10, 10, 200] weighting, the weighted terms reach comparable magnitude and the optimizer is effectively limited by the PDE residual rather than by the boundary or data terms.

Stability verdict: The loss decreases smoothly with only mild, self‑correcting Adam oscillations; no exploding gradients or divergence are observed. Convergence is essentially reached by ~50,000 – 60,000 iterations, and the remaining iterations yield only marginal improvement.

  - Selected baseline case and convergence summary

| Quantity | A3 (full-field priority) | A4 (parameter priority) |
| --- | --- | --- |
| beta error / h error | +0.472% | -0.005% |
| S error / q0 error | +0.042% | -0.351% |
| MAE | 2.175e-03 | 1.534e-03 |
| RMSE | 4.314e-03 | 4.504e-03 |
| Rel L2 | 4.410e-03 | 4.60e-03 |

A4 is adopted as the baseline. The reasoning is given in Section 1.4; this section reports its converged performance and contrasts it with the next-best candidate, A3.

A4 gives near-perfect parameter recovery (h err −0.005%, lowest MAE), so it is preferred for an inverse problem whose primary goal is identifying h and q₀. A3 attains a marginally better full-field Rel L²/RMSE, but A4's only weakness is a slightly larger local pointwise error near the switching times, as shown in the error map of Figure 1.6. This is the expected consequence of the high sensor-data weight (200) and reduced PDE weight (5): the network fits the sparse sensor data tightly at the cost of slightly weaker pointwise PDE enforcement.

The largest pointwise error occurs near the current-switching instants, where the input i(τ)² changes abruptly — the same effect documented in Section 3.3.

The FDM solution is used only to generate synthetic sensor data and to provide a post-training reference field. The inverse solver itself is the DeepXDE-based PINN trained with sparse sensor data, PDE residual, IC, and BC constraints.

Figure 1.6 — FDM reference field, PINN reconstructed field, and absolute error for the selected A4 baseline. The error is localized near the current-switching times (t = 300, 600, 900 s), while the overall transient field is well reconstructed.

The reconstructed field is physically consistent with the imposed current profile. During current-on intervals, the temperature increases due to the Joule heating source term. During current-off intervals, the temperature decays due to the convective sink term. The highest temperature region appears around the Gaussian heat source center near X=0.30, and the absolute error is mostly localized around current-switching times rather than spread over the entire domain.

2. Ablation Study

  - Evaluation metrics

Each case is compared on two axes - the result (accuracy of the recovered solution) and the convergence (training behaviour):

- Result - parameter recovery: h error (%) and q0 error (%) against the true values hTRUE=40 W/m2K and q0,TRUE=8.0×104 W/m3A2.

- Result - full field: relative L2 of the predicted theta field vs. the FDM reference (MAE / max error where useful).

- Convergence: final Loss_PDE / Loss_Total and qualitative stability of the loss curve (smooth decrease vs. high plateau or divergence).

  - Activation function ablation: Tanh vs ReLU

Only the activation is changed: To isolate the effect of the activation function, this ablation uses the same configuration as the selected A4 baseline: a 3×32 network, loss weights [5,10,10,10,200], 100k Adam iterations, num_domain=512, num_boundary=64, num_initial=64, num_test=200, and the same sparse sensor data. Therefore, any difference between the two cases can be attributed to the activation function.

Why this activation choice matters for a 2nd-order PDE: the residual needs the second spatial derivative θXX, formed by differentiating the network twice through autograd. Tanh is smooth (C∞), so second and higher derivatives are well defined. ReLU is piecewise linear, so its second derivative is zero almost everywhere - θXX collapses and the PDE residual can barely be enforced. This makes the activation-function choice physically important for a second-order heat-conduction PINN.

Setup:

To isolate the effect of the activation function, all settings are kept identical to the selected A4 baseline. Only the activation function is changed.

| Item | Control | Ablated case |
| --- | --- | --- |
| Activation | tanh | relu |
| Network | [2] + [32]x3 + [1] | identical |
| Loss weights / iterations | [5,10,10,10,200] / 100k Adam | identical |
| Collocation | num_domain/boundary/initial/test = 512/64/64/200 | Identical |
| Sensors | 3 at X=0.25,0.55,0.85; 41 time pts (123 pts) | Identical |

Results:

| Metric | Tanh | ReLU |
| --- | --- | --- |
| Final Loss_PDE | 1.804e‑03 | 1.631e+00 |
| Train / test loss | 1.97e‑03 / 8.37e‑03 | 2.58 / 2.31 |
| h error (%) | −0.005 | -100.0 |
| q0 error (%) | −0.351 | −78.179 |
| Rel L2 (full field) | 4.60e‑03 | 6.098e‑02 |

In this activation ablation, the Tanh case uses the same configuration as the selected A4 baseline. The network size, loss weights, collocation point numbers, sensor data, and training iterations are fixed, and only the activation function is changed from Tanh to ReLU.

Convergence difference: Under the identical A4 baseline setting, the Tanh model shows stable convergence, reaching a final PDE loss of 1.804×10-3 and train/test losses of 1.97×10-3 and 8.37×10-3, respectively. In contrast, the ReLU model remains at a much higher loss level, with final PDE loss 1.631, train loss 2.58, and test loss 2.31. Therefore, the final PDE loss of ReLU is about 9.0×102 times larger than that of Tanh. This indicates that the ReLU case does not simply converge more slowly; rather, it fails to represent the second-order heat-conduction physics required by the PDE residual.

Result difference: The Tanh model accurately recovers both inverse parameters. The errors are only -0.005% for h and -0.351% for q0, and the full-field relative L2 error is 4.589×10-3. In contrast, the ReLU model fails to recover the physical parameters, giving h error of -100.0% and q0 error of -78.179%. The full-field relative L2 error also increases to 6.098×10-2. Thus, the ReLU full-field error is about 13.3 times larger than the Tanh case, and the magnitude of the q0 error is more than 200 times larger.

Why ReLU fails: The failure of ReLU is physically consistent with the structure of the governing equation. The PDE residual contains the second spatial derivative θXX, which represents axial heat conduction. A Tanh network is smooth and twice differentiable, so it can provide meaningful first and second derivatives through automatic differentiation. In contrast, ReLU is piecewise linear, so its second derivative is zero almost everywhere. As a result, the ReLU network cannot properly represent the conduction term in the PDE residual. This causes the inverse parameter estimation to fail, as shown by the -100.0% error in h, the -78.179% error in q0, and the much larger full-field relative L2 error.

Figure 2.2 — Activation-function ablation under the selected A4 baseline setting. 
All training settings are identical except for the activation function. Tanh shows stable convergence, while ReLU remains at a much higher PDE-loss level because it cannot properly represent the second derivative θXX.

Tanh decreases by roughly three orders of magnitude and reaches a low PDE-loss level of 1.804×10-3, whereas ReLU rises to approximately 1.6 within the first few hundred iterations and then stays nearly flat for the remaining 100k iterations. The ReLU curve confirms the mechanism: with θXX=0 almost everywhere, the conduction term cannot be represented properly, so the PDE loss remains trapped at a high plateau. This visual gap mirrors the metric gap in the table above, where the final Loss_PDE is 1.804×10-3 for Tanh and 1.631 for ReLU.

3. Troubleshooting Experience (Flexible Reporting)

  - Scaling issue in inverse PINN training

A major difficulty during training was the scaling of the inverse problem. The physical parameters h and q₀ have very different units and magnitudes, and the Joule heating term q0It2gx can dominate the PDE residual if the equation is written directly in dimensional form, which makes the loss balance sensitive and can lead to unstable parameter updates. To resolve this, the governing equation was nondimensionalized:

X=xL,  τ=αtL2,  θ=T-T∞ΔTref,  iτ=ItIref

and, instead of directly learning h and q₀, the PINN learns the nondimensional parameters

β=hPL2kA,  S=q0Iref2L2k ΔTref

which are converted back to h and q₀ after training. This scaling keeps the network inputs and outputs near order one and prevents any single term from dominating the loss. Finally, to keep β and S physically positive throughout optimization, softplus is applied to the raw trainable variables:

β=softplusrawβ,  S=softplusrawS

  - Inconsistent synthetic data from a local-ODE approximation

Another issue was the construction of the synthetic sensor data. The initial approximation was based on a local ODE model that ignored the axial conduction term θXX, whereas the PINN is trained with the full PDE including axial conduction. This created an inconsistency between the data loss and the physics loss. To fix it, the synthetic sensor data were regenerated from a finite-difference solution of the full nondimensional PDE (nx = 101, adiabatic ends), and the sensor values used in PointSetBC were extracted from this FDM reference field. The data constraint is now consistent with the PDE residual used in training, and the same FDM field is reused for the full-field comparison.

  - Remaining mismatch near current-switching times

A remaining limitation is the mismatch near current-switching times. Even when the learned inverse parameters are close to the true values, the PINN temperature field can deviate from the FDM reference near abrupt changes in the current input. This is likely because the current profile is piecewise constant while the network represents the temperature field as a smooth function, so it cannot resolve the sharp temporal transition as tightly as the FDM grid. For this reason the full-field FDM comparison was added as an essential verification step (Section 1.6). Planned improvements include adding more collocation points near the switching times, slightly smoothing the current profile, and comparing different current profiles in future validation tests.

  - Loss-weight balancing and inverse parameter identifiability

The weight sweep itself is in Section 1.4; here we focus on why it matters for identifiability. A practical issue was the balance between the physics loss and the sensor-data loss. In the inverse PINN, the PDE residual enforces the heat equation, but the unknown parameters β and S cannot be accurately identified unless the sparse sensor data provide a strong enough constraint. In the early loss-weight trials, increasing the PDE or boundary-condition weights alone improved some parts of the physics residual but did not consistently improve parameter recovery.

The loss-weight study showed that the data loss weight was especially important for parameter identifiability. When the sensor-data weight was increased to 200 while keeping a moderate PDE weight, the model achieved near-perfect recovery of β and h, with only a small error in S and q0. Therefore, the case [5, 10, 10, 10, 200] was selected as the baseline.

This result suggests that, for this inverse problem, the sensor data mainly determine the unknown thermal parameters, while the PDE and boundary-condition losses regularize the solution and enforce physical consistency. If the data weight is too small, the network may satisfy the PDE but fail to recover the correct parameters. If the data weight is too large, the model may fit the sensor points tightly but show slightly larger local full-field errors. Therefore, the final baseline was chosen as a compromise between parameter accuracy and full-field reconstruction accuracy.